WBS 1.6 Software

- Remaining Software Task-list: A draft task-list of tasks and associated manpower (or needed manpower) for completing the HFT software and Run14 readiness.

- Full System Simulations: Further preparatory work to do a detailed full system simulation for KPP and physics performance is progressing well, including better and more realistic geometry and pileup estimates.

- Alignment:

  - Code has been developed to perform iterative, PXL Sector alignment based on LHC-CMS work. The new element is that alignment parameters can be determined for a Sector as a single object.

  - An “Alignment” challenge was performed on a Simulation sample with an unknown mix of rotations and translations. Both sector-based and ladder-based approaches reported results within 20 microns from input and 0.2mrad in rotations (corresponding to less than 20 microns maximum shifts). The Table below summarizes the reported results:

|  | d-alpha (mrad) | d-beta | d-gamma | dx (mm) | dy | dz |
| --- | --- | --- | --- | --- | --- | --- |
| Ladder-based | -5+-0.2 | 7.4+-0.2 | 13+-1 | NA | NA | 115+-15 |
| Sector-based | -5.0 | 7.3 | 13.3 | -710 | 508 | 118 |
| Input | -5 | 7.47 | 13.0 | -750 | 500 | 100 |

  - A similar challenge was performed on real Data with very encouraging results.

- PXL and IST software review:  The PXL software review is progressing and the IST packages are undergoing an internal review before submission to STAR.

- AOB

  - We participated in a half a day HFT meeting during  the STAR collaboration meeting at LBNL.

  - Some modifications to the Pxl simulation structures were completed and tested.