STAR Heavy Flavor Tracker (HFT)

Response to CD-1 Physics Questions

July 31, 2026

# Introduction

This report is written in response to the questions raised in the DOE document “ Report on the Technical, Cost, Schedule and Management Review of the STAR Heavy Flavor Tracker". This review took place at Brookhaven National Laboratory in November 12-13, 2009, and is referred to here as the CD-1 review.

In the first part we describe the simulations that have been performed since the review and then address all the relevant questions.

In the Appendix we present simulations where we compare the significance of D0 measurements when using HFT pixel technology versus PHENIX hybrid technology.

# New HFT Simulations

The CD-1 questions emphasize the impact on physics of the HFT design parameters, especially in the low pT region that is very sensitive to detector thickness. In this report we study the impact of increasing the mass of the first layer of the PXL detector from a value of 0.32 % of a radiation length (X0) (thin configuration) to a value of 0.62 % X0 (thick configuration), and of increasing the internal stability from 20 mm (design value) to 30 mm (CD-4 parameter). The thickness of the “thin” configuration is close to the present design parameter of 0.37% X0, while the value for the “thick” configuration is close to what could be achieved by using Cu cables for the PXL readout and is also close to the CD-4 parameter for the thickness of the first pixel layer.

All simulations in this report where performed in the same environment as the ones included in the CDR and CD-1 presentations, i.e. the standard STAR simulation and reconstruction chain is used as well as the same HFT detector configuration (geometry). The HFT geometry used in all simulations (slightly different than the latest proposed design) comprises of two layers of PIXEL detectors at 2.5 and 8 cm radius, one IST layer of 600 (r-) x 6000 (z) micron strip-lets at a radius of 14 cm and the existing SSD detector at 23 cm radius.

To summarize the additional simulation efforts after the CD-1 review:

- We ran new productions for thin/thick scenarios to more than quadruple our available statistics in the low pT region.

- We performed cut-optimization studies for enhanced low pT D0 significance, resulting in a gain of efficiency of about a factor of two at low pT.

- We re-evaluated our capability to measure the LC /D0 ratio.

# Answers to CD-1 questions

•   Studies should be carried through to the final physics measurement, showing the degradation of the final physics significance if key requirements are not met:

–Give an explicit evaluation of what the loss in low-pT efficiency does regarding the fundamental physics questions relating to flow and energy loss of heavy quarks in the hot-dense medium.  Evaluate this loss in terms of current theoretical models and show whether these are well tested by the measurement above ~2 GeV/c or if the loss of statistics at lower pT is a critical loss.

The low pT region is important for studies of collectivity. Energy loss is a high pT phenomenon. Here we limit our discussion to v2, for which the low pT region is relevant. It is important to understand that flow is a hydrodynamic phenomenon. Data and hydro predictions agree up to about 1.5 GeV/c pT beyond which point data deviate dramatically from the hydro prediction. In Figure 1 we present the precision of measurement that can be achieved with the thin and the thick PIXEL configuration within the first year of data taking. Also shown in the figure are two fifferent model calculationn. based on the coalescence assumption. Coalescence is an empirical observation deduced from v2 systematics. At lowepT we observe mass scaling/splitting (hydro) and at high pT leveling off and scaling with the number of constituent quarks. The shape of v2 at high pT is not a hydro effect but is due to quark energy loss. Reference Error: Reference source not found has no predictive power at high pT (above 2 GeV/c) since it only assumes coalescence and does not include the effects of energy loss mechanism. The model uses light quark momentum distributions and for the heavy quark either non-interacting distributions (no flow) or completely thermalized distributions with transverse expansion (flow). In fact, if constituent quark scaling is to hold in the charm sector, the v2 value at high pT is determined only by the fact that the D0 is a meson (not a baryon). In this scenario, the two theoretical curves must merge at high pT. There is no realistic model on the market that would give quantitative guidance for v2 values at low pT and realistic predictions for v2 at high pT. Therefore, a comprehensive comparison with theory predictions will not be possible, unless we see a new description emerge. The comparison has to rely on systematic studies of v2 scaling at low pT as a function of particle mass. If the heavy quark flows, such systematics will show it. Figure 1 also shows data for charged hadrons. The measured v2 values do not saturate, but decrease at high pT. This is further evidence that the calculations from Reference [Error: Reference source not found] are not realistic at high pT.

However, this further emphasizes the importance of testing coalescence in the charm sector. Besides v2 scaling, the LC to D0 cross-section ratio represents an ideal test. Later in this document (section 3.1) we summarize the capability of the HFT to determine a baryon to meson ratio in the charm sector.

Figure 1:  Elliptic flow (v2) vs pT in Au+Au collisions at 200 GeV. The purple curve shows the measured value for charged hadrons. The red and the green curves show calculations from Ref [Error: Reference source not found] for the limiting cases that the charm quark flows like the light quarks and that the charm quark does not flow.  The Cyan band indicates the statistical error that can be achieved with 500 M minimum bias events for the thin detector configuratios and the yellow band is for the thick configuration.

We turn now the discussion to the question of ‘the degradation of the final physics significance if key requirements are not met’. There are two items of uncertainty to consider here. One is the increase of the PXL internal vibrational instability from 20 to 30 microns and the other is the increase of PXL thickness due to Cu instead of Al cable traces. The effect of the first one is discussed elsewhere. The effect of the latter one is a degradation of the single track DCA resolution from 35 (thin) to 45 (thick) microns for kaons of 750 MeV/c. (Note that both values are significantly less than the CD-4 requirement of 50 microns). This, for the actual thickness increase, would imply a required increase in the number of events of about a factor of 2-2.5 in order to keep the signal significance the same (see Figure 3 and Reference [7]).  Figure 2 (filled and open circles and squares) shows the change in significance due to thickness. The same figure (open triangles and stars) shows a new result, the signal significance for thin/thick but after a careful cut optimization was performed in the lower pT region. The net gain from this optimization (about a factor of two) is shown in essA potential increase in thickness would not change the precision of low pT values established in the CDR , since a factor of two in efficiency has been gained by tuning our low pT cut parameters. At this point it outnoted that our analysis procedure is not yet fully optimized andturrtere gains in efficiency might be expected in the futurted.

Figure 2:  Signal significance as a function of pT. Filled/open circles/squares shows the change in significance due to thickness (thin/thick) for flat/realistic spectral shape. Open triangle/stars show a new result, the signal significance for thin/thick after a careful cut optimization was performed in the lower pT region.

Figure 3:  Impact of PXL thickness on signal significance as a function of pT. The different symbols represent different pT shapes and/or optimized cuts, as labeled.

Figure 4:  Signal significance gain as a function of  pT after cut optimization for thin (black symbols) and thick (red symbols).

•    Compare the significance of planned charm and beauty measurement to be done with the HFT to similar measurements expected from the upgraded PHENIX detector.  Comment on how significant an advance in theoretical understanding of energy loss and flow for the hot-dense medium the HFT would provide compared to the earlier anticipated PHENIX measurements

PHENIX has not shown simulations that would establish the capability to do topological reconstruction of D-mesons.,5 Therefore we will attempt a qualitative comparison of what can be done with the HFT and PHENIX with respect to the physics extracted from the measurement of the electrons from semi-leptonic D- and B-meson decays. We believe the following statements to be correct:

- The theory development in the area of energy loss is progressing rapidly and it is not obvious what it will be in a few years. It is, however, safe to say that quality data are the requirement for theory progress.

- All measurements by STAR based on topological reconstruction are original and without competition at RHIC.

- PHENIX and STAR will both measure charm/beauty production cross sections from the electron spectra.

PHENIX will measure the spectra of non-photonic electrons from charm and beauty decays. The new information that the HYBRID vertex detectors will provide is the electron impact parameter (DCA) from the event vertex. The reconstruction technique is based on applying DCA cuts to reject background and then fit the yields in various pT bins. This will result in a spectrum of the sum of D and B decays. Taking the difference in c for charm and bottom into account, separation can be achieved through unfolding. The unfolding process is complicated by the fact that the different D states have quite different ct values, where for example the D+ c is very close to the B c. Unfolding has to make assumptions about the production ratio for the individual D states. For p+p this is well known. However, in case that the production ratio is modified in heavy ion reactions, unfolding becomes unreliable.

This potentially large uncertainty can be mitigated in the STAR non-photonic electron measurement because STAR will directly measure D0, D+, and Ds production.

D and B production can be separated also by multi-particle correlations. Neither PHENIX nor STAR have shown this capability in simulations. Since 2 acceptance is important for two- and multi-particle correlations, it can be assumed that STAR has advantages for this particular measurement.

PHENIX will perform a measurement of the RAA for electrons from D and B decay as a function of pT. Due to momentum smearing from the decay process, the parent pT cannot be determined, thus preventing a precision measurement of RAA for heavy flavor hadrons as a function of pT, which might be very important for a precision comparisons with model calculations which might be important to determine the mechanism of energy loss.

PHENIX also will determine v2 of non-photonic electrons. This measurement cannot contribute to the question of thermalization. The parent pT is not determined to better than 3 GeV. We have argued above that only a measurement at low pT (< 2.5 GeV/c) might be able to answer this important question.

In summary, a measurement of non-photonic electrons has a very limited reach. The full potential of heavy flavor physics in heavy ion collisions can only be reached through full topologic reconstruction.

## Update on LC Simulations

Since CDR we have increased statistics of our simulations, to allow for better optimization of cuts also in the LC analysis. Despite these improvements, estimated errors in the 2-3 and 3-4 GeV/c pT bins haven't changed significantly, showing robustness of our CDR estimates.

A significant improvement was achieved in the 4-5 GeV/c pT bin, where we didn't require full identification of daughter particles, which resulted in improvements in LC reconstruction efficiency and (as background is modest in this higher pT bin) increased the LC signal significance.

Note that in the figure, the discrimination should be made between estimated errors and the 2 scenarios of LC /D0 ratio - not between the two sets of estimated errors. The significance of this discrimination is in the range 2-4 sigma in the case of an enhanced ratio and about 4-6 sigma in the extreme case of no-enhancement.

Similar simulations and analysis of simulated data were conducted for the "thick" detector configuration. However, LC /D0 measurement with similar errors turned out not to be feasible with reasonable statistics in the "thick" detector configuration.

In summary, we will be able to make a significant measurement of the LC /D0 ratio only with the thin HFT configuration. This measurement will become much harder with the thick detector configuration.

Figure 5:  Expected statistical errors of the ratio of LC to D0 production for different assumptions about the production mechanism.

- Appendix:

We have also performed simulations of the significance of D0 spectra for the case where we use the PHENIX hybrid technology (ie. pixel dimensions) instead of the HFT pixel technology. We did not change the detector thickness or the geometrical acceptance. Figure 6 shows the ratio of significance for pixel versus hybrid technology as a function of pT. For a given number of events the significance of a HFT measurement is about a factor of 10 better. This means that in order to achieve the same significance as STAR, PHENIX has to acquire a factor of hundred more events. Important factors, like partial (PHENIX) acceptance, and bandwidths  has not yet been factored in into this rough estimate.

Figure 6:  Gain in significance by using HFT PXL technology versus PHENIX hybrid technology.

- References:

V. Greco, C.M. Ko and R. Rapp, Phys. Lett. B 595 (2004) 202

Coalescence refs

G.D. Moore and D. Teaney, Phys. Rev. C 71, (2005) 064904

We need a reference here. If not we should drop the complex.

Reference to CDR

W. Zajc, private communication

STAR publication

6 PHENIX Silicon Vertex Upgrade Review, June 2, 2009

7 ‘Answers to Questions’ by H. Wieman, CD1 review